Department of Computer Science & Engineering (AI & ML)

Microprocessor & Computer Architecture Lab

Lab 1 Programs

UE23CS251B

| 1 | Write an ALP to perform Addition for of two numbers of size a)64 bit b)128 bit save the result in register (reuse the register to store the result) Hint: For 128 bit given below R3 R2 R1 R0 +R7 R6 R5 R4 a) .text .global _start _start: LDR R0, =0x12345678 LDR R1, =0x87654321 LDR R2, =0x11111111 LDR R3, =0x22222222 ADDS R0, R0, R2 ADC R1, R1, R3 b) .text .global _start_128 _start_128: LDR R0, =0x11111111 LDR R1, =0x22222222 LDR R2, =0x33333333 LDR R3, =0x44444444 LDR R4, =0x55555555 LDR R5, =0x66666666 LDR R6, =0x77777777 LDR R7, =0x88888888 ADDS R0, R0, R4 ADCS R1, R1, R5 ADCS R2, R2, R6 ADC R3, R3, R7 |
| --- | --- |
| 2 | Write an ALP to perform 2’complement only using mov and RSB instruction. .section .text .global _start _start: MOV R0, #-7 @ Load -1 into R0 RSB R1, R0, #0 @ R1 = 0 - R0 |
| 3 | Write an ALP to perform not operation only using mov and bitwise logical instructions. .section .text .global _start _start: @perform not operation only using mov and bitwise logical MOV R1, #0xFFFFFFFF ; Load all 1's @ assumed R0 is loaded with 0 LDR R0, =0x99999999 ; Load some random value EOR R2, R0, R1 |
| 4 | Write an ALP to subtract if the numbers are equal, otherwise add them. .text .global _start _start: MOV R0, #20 @ Load 20 into R0 MOV R1, #10 @ load 10 into R1 CMP R0, R1 @ Compare R0 and R1 BEQ subtract ADD R2,R0,R1 @ R2 = R0 + R1 B end subtract: SUB R2,R0,R1 @ R2 = R0 - R1 end: |
| 5 | Check if a given number is even or odd. Note: at the end of the program execution R2 contains 0 if number is even, otherwise R2 contains 1. .text .global _start _start: MOV R0, #21 @ Load number to check AND R1, R0, #1 @ R1 = R0 & 1 (remainder when divided by 2) MOV R2, R1 @ Move result to R2 (0 for even, 1 for odd) |
| 6 | Write a program to find the factorial of a given number. .text .global _start _start: MOV R0, #5 MOV R1, #1 factorial_loop: CMP R0, #1 BLE factorial_end MUL R1, R0, R1 @ r1 = r1 * ro SUB R0, R0, #1 B factorial_loop factorial_end: |
| 7 | Write a program to find GCD of two numbers. @to find gcd of 2 numbers .text .global _start _start: MOV R0, #80 @ Load 20 into R0 MOV R1, #90 @ load 10 into R1 gcd_loop: CMP R0, #0 @ Compare R0 and R1 BEQ gcd_end CMP R0,R1 BLT swap SUB R0,R0,R1 B gcd_loop swap: MOV R2,R0 MOV R0,R1 MOV R1,R2 B gcd_loop gcd_end: |